Lv: SPRINKLER IRRIGATION SYSTEM

1.	Titel: SETTING UP THE BUILDING SITE

1.1.	furnishing the building site

Clearing and furnishing the building site,

placing of equipment on site which is

required to carry out all work as detailed in

the performance bill.

1,00	LUMP SUM	_______________	__________________

Summe Titel 1. SETTING UP THE BUILDING SITE	__________________

2.	Titel: EARTH WORK

*Eventualposition

2.1.	lifting the turf sods

The contractor shall lift the turf sods

on the sportsfield.

The turf sods have to be placed alongside the

trench, and to be placed back again.

1,00	m	_______________	nur Einheitspreis

2.2.	Excavation trenches up to 0,80m

The contractor shall excavate the trenches for

the water supply pipe lines according to

construction practice. The soil is to be

placed alongside the trench. Backfilling

according to instruction sheet for

backfilling of pipe line trenches. Soil

displacement more than 5 to 10%.

Surplus soil to be leveled alongside the trench.

Excavation depth up to 0,80 m.

Floor width of trenches up to 0,50 m.

Class soil 4 - DIN 18300

500,00	m	_______________	__________________

*Eventualposition

2.3.	Road Crossings

Where the pipe crosses an existing or

proposed road, the pipe shall be sleeved in a

concrete pipe extending 3 meters clear of the

road on either side. The sleeve pipe shall be at

least 75 mm larger (internal diameter) than the

outside of the pipe, cables should be placed

alongside in a steel or PVC conduit and a

pull cord left for any future work.

1,00	m	_______________	nur Einheitspreis

*Eventualposition

2.4.	Deposited soil

Deposited surplus soil, has to be loaded, transported

and leveled within the site boundary. Class

of soil 4 - DIN 18300 -Distance of

transportation up to 500 m. Amount of excess

soil from afore listed positions.

1,00	cbm	_______________	nur Einheitspreis

2.5.	Excavation of pits for sprinkler

Excavation of pits for pop up sprinkler

connections and hydrants from the top of the

area surface. Deposit soil alongside the

pits, backfilling and compaction according to

instruction sheet for back filling of pipe

line trenches.

Soil displacement more than 40-50%.

Surplus soil to be leveled alongside the pits.

Excavation depth up to 0,80 m.

Floor width of trenches more than 0,5 m up to

1,0 m.

Class of soil 4.

12,00	pc.	_______________	__________________

*Eventualposition

2.6.	Backfilling material

Backfilling material - Installation.

If the material available for backfilling

contains stones, rocks or other debris which

may damage pipe, cable or tubing it may be

necessary to import suitable backfilling

sand. The supply of such material will be the

responsibility of the contractor.

Layer thickness up to 5 cm.

Floor width as described above.

1,00	m	_______________	nur Einheitspreis

2.7.	Gravel packing for pop up sprinklers

Installation of a gravel packing for

stabilizing and draining of the pop up

sprinklers.

Material: coarse gravel.

Layer thickness approximately 20 cm.

12,00	pc.	_______________	__________________

Summe Titel 2. EARTH WORK	__________________

3.	Titel: PIPE LINES AND CABLES

3.1.	Water supply pipe PN12,5/PE-HD Dia 75mm

Supply pipe installed from the pumpstation or waterconnecting point to the ring pipe.

Material to be PE hard as per

DIN 19533 - PN12,5/PE-HD. Outside DIA 75 mm;

Wall thickness 6,9 mm; Weight 1,48 kg/m.

including all pipe fittings made of plastic

as per DIN 8076.

50,00	m	_______________	__________________

3.2.	Ring pipe PN12,5/PE-HD Dia 63mm

Pipes installed along the edge of the sportsground and to the sprinklers in the midfield, according BA10120.

Material to be PE hard as per

DIN 19533 - PN12,5/PE-HD. Outside DIA 63 mm;

Wall thickness 5,8 mm; Weight 1,05 kg/m

including all pipe fittings made of plastic

as per DIN 8076.

T-piece has to be used for the connections of the

swing joints, saddle clamps are not allowed.

450,00	m	_______________	__________________

3.3.	Controlcable

Control cable

Cross section 2x2,5 sq.mm NYY-O with double

insulation, for underground installation

according to the rules and regulations of the

local Energy Supply Commission. Cable

connections to be made with water tight

splicing kits. Cable is installed along the

main-line from the digital controller to each sprinkler

block according BA10120.

750,00	m	_______________	__________________

3.4.	Connection  from supply pipe dia 75

to the ring pipe dia 63 mm

Connection of ring pipe with dia 63 mm with clamp fittings made of plastic as per DIN 8076 consisting of:

1 pc. 	clamp T-connector with female thread,

type 63 x 2” x 63

1 pc. 	clamp connector male threaded 75 x 2”

1,00	pc.	_______________	__________________

3.5.	Connection for sprinkler DN 40

to the PE -pipe dia 63 mm for the sprinkler at the edge of the field.Connection with clamp fitting made of plastic

as per DIN 8076 Consisting of:

1 pc. 	clamp T-connector with female thread,

type 63 x 1 1/2” x 63

10,00	pc	_______________	__________________

3.6.	Connection for sprinkler DN 40

to the PE -pipe dia 63 mm for the sprinkler in the center field.

Connection with clamp fitting made of plastic as per DIN 8076

Consisting of:

1 pc. clamp connector with female thread,  type 63 x 1 ½”

2,00	pc.	_______________	__________________

Summe Titel 3. PIPE LINES AND CABLES	__________________

4.	Titel: SPRINKLERS

4.1.	Full circle pop up sprinkler LVZR 22SVAC

Full circle pop up sprinkler installed in the midfield

according BA10120.

High rotation speed of less than one minute for 360°.

This feature may be evidenced.

Sprinkler with 3 nozzles

and service-free impact lever drive,

22° trajectory.

Cover with green (brown) plastic grass

Flow rate 16m³/h at an

operating pressure of 5,5bar at the

sprinkler.

Casting range 28,2m

Electric valve-in-head. Coil and valve are

removable from the top, without any excavation work

Type: Perrot LVZR 22 SVAC  13 x 5,5 x 5,2

2,00	pc.	_______________	__________________

*Alternativposition

4.2.	Perrot Hydra L-SVAC Full-circle Gear-driven Pop-up Sprinkler

Full circle pop up sprinkler installed in the midfield

according BA10120.

Maintenance free gear driven sprinkler with closed housing and built-in electrical control valve, pressure compensated.

Flow rate 17,8m³/h at an

operating pressure of 5,5bar at the

sprinkler.

Casting range: 30m

Connection 1½“

All parts like valve, dirt filter and pressure regulator with solenoid can be exchanged from above without digging.

Type: Perrot Hydra L-SVAC 13

2,00	pc	_______________	nur Einheitspreis

4.3.	Part circle pop up sprinkler LVZR 22WVAC

Part circle pop up sprinkler installed at the edge

of the sportfield according BA10120.

High rotation speed of less than one minute for 360°.

This feature may be evidenced.

With 2 nozzles and service-free impact lever drive,

22° trajectory,with green (brown) plastic cover

infinately adjustable from

30° to 360°, manual without tools.

Left and right sector must be adjustable.

Flow rate 9 m³/h at an operating pressure of 5,5 bar

at the sprinkler.

Casting range 23 m

Electric valve-in-head.

Coil removable from the top, without any excavation work.

Type: Perrot LVZR 22 WVAC  9 x 5,5

10,00	pc.	_______________	__________________

*Alternativposition

4.4.	Perrot Hydra L-WSVAC Part-circle Gear-driven Pop-up Sprinkler

Part circle pop up sprinkler installed at the edge

of the sportfield according BA10120.

Maintenance free gear driven sprinkler with closed housing and built-in electrical control valve,pressure compensated.

Connection 1½“

Infinetely adjustable for part circle setting from 30° to 360°.

Left and right sector must be adjustable, also when at rest; the configured sector is displayed on the circular scale.

Flow rate 9,4m³/h at an operating pressure of 5,5bar

at the sprinkler.

Casting range: 26m

Connection 1½“

All parts like valve, dirt filter and pressure regulator with solenoid can be exchanged from above without digging.

Type: Perrot Hydra L-WSVAC 9

10,00	pc	_______________	nur Einheitspreis

4.5.	Swing joints size 1½"

Articulated riser swing joints for all pop up

sprinklers as detailed before.

Size 1½"

12,00	pc.	_______________	__________________

Summe Titel 4. SPRINKLERS	__________________

5.	Titel: CONTROL SYSTEM

5.1.	Controller Perrot WaterControl

Digital Controller, no.of stations variable by output

moduls from 2 to 12.

Automatic and manually operation for programs

and single stations.

3 programs with 4 start times each.

7 day, weekly shedule with automatic repeat.

0 to 99 minute run time per station.

0 to 99 sec. break time between the stations

24VAC permanent output

Output for master-valve and rainsensor.

Short circuit protected by multifuse.

Power supply with 240V/50Hz.

300 mA output current max.

Cable entrance with plugs and strain relief clamps.

Housing for wallmounting, IP54 protection

LCD display wit selfexplaining programsymbols.

Indication of time, weekday, runtime per station

and per program alternating.

Type: PERROT WaterControl 8AC

1,00	St	_______________	__________________

*Eventualposition

5.2.	Rain sensor Mini Click I

Automatic rain sensor which fits the above

described Perrot controller. Installation

nearby the club house and connection to

the controller.

Type: Mini-Click I

1,00	pc.	_______________	nur Einheitspreis

Summe Titel 5. CONTROL SYSTEM	__________________

6.	Titel: PUMP STATIONS

6.1.	Vertical Pump Station

Fully automatic pump station with vertically

arranged rotary pump, build on a Niro-pedestal.

Completly installed with required suction and pressure

pipe connections, including required gate- and non return

valves.

18 Ltr. Membrantank with pressure switch.

Unit is completly wired and adjusted for operation.

Pump type:  GRUNDFOS CR 16-80

capacity of the pump:              		20  m³/h

delivery head :                     		82  m

motor output - per pump:             	7,5KW

max. current consumption - per pump 	15  A

voltage :                          		400  V

Supply and installation into the pump house

which has to have the following minimum

dimensions (LxWxH) 3,5x2,5x2,5m.

The installation is carried out with

rustproof supporting and mounting elements.

1,00	pc.	_______________	__________________

Summe Titel 6. PUMP STATIONS	__________________

7.	Titel: MAIN VALVES, VALVES,HYDRANT,DRILLINGS

7.1.	Flanged Gate Valve DN65

Flanged gate valve with installation device

for the installation in the supply pipe.

Supply and installation -ready to use-

as follows: Beta Valve DIN 3352 - DN 65

including connections. Flanged

socket adaptor with threaded connections.

Street top, carrier plate, stabilizer plate,

valve linkage unit.

1,00	pc.	_______________	__________________

7.2.	Drain Assemblies for Main pipes

Drain assemblies for main pipe lines

(DN 65-110) including coverage with a valve box

as well as all fittings, earth and additional

work.

1,00	pc.	_______________	__________________

Summe Titel 7. MAIN VALVES, VALVES,HYDRANT,DRILLINGS	__________________

Summe Lv  SPRINKLER IRRIGATION SYSTEM	__________________

Titel 1. SETTING UP THE BUILDING SITE	__________________ EUR

Titel 2. EARTH WORK	__________________ EUR

Titel 3. PIPE LINES AND CABLES	__________________ EUR

Titel 4. SPRINKLERS	__________________ EUR

Titel 5. CONTROL SYSTEM	__________________ EUR

Titel 6. PUMP STATIONS	__________________ EUR

Titel 7. MAIN VALVES, VALVES,HYDRANT,DRILLINGS	__________________ EUR

Gesamt netto	__________________ EUR

zzgl. 16 % MwSt 	__________________ EUR

Gesamt brutto	__________________ EUR

Ort/Datum/Stempel/Unterschrift