Lv: SPRINKLER IRRIGATION SYSTEM

1.	Titel: SETTING UP THE BUILDING SITE

1.1.	furnishing the building site

Clearing and furnishing the building site,

placing of equipment on site which is

required to carry out all work as detailed in

the performance bill.

1,00	LUMP SUM	_______________	__________________

Summe Titel 1. SETTING UP THE BUILDING SITE	__________________

2.	Titel: EARTH WORK

2.1.	Excavation of trenches up to 0,60m

The contractor shall excavate trenches for

the water supply pipe lines according to

sound construction practice. The soil is to

be placed alongside the trench. Backfilling

according to instruction sheet for

backfilling of pipe line trenches.

Soil displacement more than 5 to 10%.

Surplus soil to be leveled alongside the trench.

Excavation depth up to 0,60 m.

Floor width of trenches up to 0,50 m.

Class soil 4 - DIN 18300

125,00	m	_______________	__________________

*Eventualposition

2.2.	Road Crossings

Where the pipe crosses an existing or

proposed road, the pipe shall be sleeved in a

concrete pipe extending 3 meters clear of the

road on either side. The sleeve pipe shall be at

least 75 mm larger (internal diameter) than the

outside of the pipe, cables should be placed

alongside in a steel or PVC conduit and a

pull cord left for any future work.

1,00	m	_______________	nur Einheitspreis

*Eventualposition

2.3.	Deposited soil

Deposited surplus soil, has to be loaded, transported

and leveled within the site boundary. Class

of soil 4 - DIN 18300 -Distance of

transportation up to 500 m. Amount of excess

soil from afore listed positions.

1,00	qbm	_______________	nur Einheitspreis

2.4.	Excavation of pits for sprinkler

Excavation of pits for pop up sprinkler

connections and hydrants from the top of the

area surface. Deposit soil alongside the

pits, backfilling and compaction according to

instruction sheet for back filling of pipe

line trenches.

Soil displacement more than 40-50%.

Surplus soil to be leveled alongside the pits.

Excavation depth up to 0,80 m.

Floor width of trenches more than 0,5 m up to

1,0 m.

Class of soil 4.

6,00	pc.	_______________	__________________

*Eventualposition

2.5.	Backfilling material

Backfilling material - Installation.

If the material available for backfilling

contains stones, rocks or other debris which

may damage pipe, cable or tubing it may be

necessary to import suitable backfilling

sand. The supply of such material will be the

responsibility of the contractor.

Layer thickness up to 5 cm.

Floor width as described above.

1,00	m	_______________	nur Einheitspreis

2.6.	Gravel packing for pop up sprinklers

Installation of a gravel packing for

stabilizing and draining of the pop up

sprinklers.

Material: coarse gravel.

Layer thickness approximately 20 cm.

6,00	pc.	_______________	__________________

Summe Titel 2. EARTH WORK	__________________

3.	Titel: PIPE LINES AND CABLES

3.1.	Supply pipe PN12,5/PE-HD Dia 50mm

Pipes installed from the pumpstation or waterconnecting point to the valve box outside the tennis court.

Material to be PE hard as per

DIN 19533 - PN12,5/PE-HD.

Outside DIA 50 mm; Wall thickness 4,6mm;

Weight 0,67 kg/m including all pipe fittings

made of plastic as per DIN 8076.

30,00	m	_______________	__________________

3.2.	Distribution pipe PN7,5/PE-HD Dia 32mm

Pipes installed as distribution pipe from

the solenoid valves to the individual pop up

sprinkler, according BA 10125.

Material to be PE hard as per

DIN 19533 - PN7,5/PE-HD. Outside DIA 32 mm;

Wall thickness 1,9 mm; Weight 0,19 kg/m.

Including all pipe fittings made of plastic

as per DIN 8076.

95,00	m	_______________	__________________

3.3.	Controlcable 3x2,5sq.mm

Control cable

Cross section 3x2,5 sq.mm NYY-O with double

insulation, for underground installation

according to the rules and regulations of the

local Energy Supply Commission. Cable

connections to be made with water tight

splicing kits. Cable is installed along the

main-line from the controller to each single valve.

50,00	m	_______________	__________________

3.4.	Controlcable 2x2,5sq.mm

Control cable

Cross section 2x2,5 sq.mm NYY-O with double

insulation, for underground installation

according to the rules and regulations of the

local Energy Supply Commission. Cable

connections to be made with water tight

splicing kits. Cable is installed along the

pipe from the valve to the short interval time clock.

50,00	m	_______________	__________________

Summe Titel 3. PIPE LINES AND CABLES	__________________

4.	Titel: SPRINKLERS

4.1.	Part circle pop up sprinkler LVZA 22WT

Part circle pop up sprinkler with 1 nozzle

and service-free impact lever drive,

installed at the edge of the tennis court,according BA10154.

22° trajectory,

infinately ajustable for part circle setting from 30°

to 360° -manual without tools.

Water consumption 1,2 m³/h at an

operating pressure of 4 bar at the

sprinkler.

Casting range 12,5 m

Type: Perrot LVZA 22 WT

6,00	pc.	_______________	__________________

4.2.	Swing joints size 1"

Articulated riser swing joints for all pop up

sprinklers as detailed before.

Size 1"

6,00	pc.	_______________	__________________

Summe Titel 4. SPRINKLERS	__________________

5.	Titel: SOLENOID VALVES, DECODER, VALVE BOXES

5.1	Solenoid valve MVR - 1"AC

Solenoid valve with housing made of brass

Normaly closed. Manually on/off,

special prepared filter for control water

prevents valve from failure

Max. pressure 12.5 bars

24 VAC and 0.13 A inrush current.

Size 1"

Typ: PERROT MVR-1"

Including all connections with approved water

Tight cable joint. To be installed with a PE

pipe loop, bend up to appr. 0.15m

below the top of the valve box lid.

1,00	pc.	_______________	__________________

5.2.	Ball Valve 1"

Threaded gate valve for shutting off the

various irrigation units,

Size 1"

Installation in front of the solenoid valve.

1,00	pc.	_______________	__________________

5.3.	Valve box VBJ 350 x 500

Valve box with extention if necessary.

Measurements (L,W) 50 x 35 cm. Depth

depending on local conditions, complete with a

locable lid, to be installed at the required

depth and flush on the gravel packing at the

above listed positions, including all

necessary pipe and cable passages, small and

securing parts as well as earthing up to

facilitate moving procedures. Installation

over all solenoid valves and water take off

points.

Type: VBJ 350 x 500

1,00	pc.	_______________	__________________

Summe Titel 5. SOLENOID VALVES, DECODER, VALVE BOXES	__________________

6.	Titel: CONTROL SYSTEM

6.1	Controller Perrot WaterControl

Digital Controller, no.of stations variable by output

moduls from 2 to 12.

Automatic and manually operation for programs

and single stations.

3 programs with 4 start times each.

7 day, weekly shedule with automatic repeat.

0 to 99 minute run time per station.

0 to 99 sec. break time between the stations

24VAC permanent output

Output for master-valve and rainsensor.

Short circuit protected by multifuse.

Power supply with 240V/50Hz.

300 mA output current max.

Cable entrance with plugs and strain relief clamps.

Housing for wallmounting, IP54 protection

LCD display wit selfexplaining programsymbols.

Indication of time, weekday, runtime per station

and per program alternating.

Type: PERROT WaterControl 12AC

1,00	St	_______________	__________________

6.2.	Short interval time clock

For manual start of the irrigation directly on the the tennis court. Adjustable interval time from 0-15 min.

Installation at the entrance of the tennis court.

Supply, installation and wiring of the timer.

Type: Jung-Short Interval Time Clock

1,00	St.	_______________	__________________

*Eventualposition

6.3.	Rain sensor Mini Click I

Automatic rain sensor which fits the above

described Perrot controler. Installation

nearby the club house and connection to

the controller.

Type: Mini-Click I

1,00	pc.	_______________	nur Einheitspreis

Summe Titel 6. CONTROL SYSTEM	__________________

7.	Titel: PUMP STATIONS

7.1.	Vertical Pump Station

Fully automatic pump station with vertically

arranged rotary pump, build on a Niro-pedestal.

Completly installed with required suction and pressure

pipe connections, including required gate- and non return

valves.

18 Ltr. Membrantank with pressure switch.

Unit is completly wired and adjusted for operation.

Pump type:  GRUNDFOS CR 8-60

capacity of the pump:              		7  m³/h

delivery head :                     		60  m

motor output :  			           	2,5KW

max. current consumption - per pump 	 5  A

voltage :                          		400V

Supply and installation into the pump house

which has to have the following minimum

dimensions (LxWxH) 3,5x2,5x2,5m.

The installation is carried out with

rustproof supporting and mounting elements.

1,00	pc.	_______________	__________________

Summe Titel 7. PUMP STATIONS	__________________

8.	Titel: MAIN VALVES, VALVES,HYDRANT,DRILLINGS

8.1.	Flanged Gate Valve DN50

Flanged gate valve with installation device

for the installation in the main pressure

line. Supply and installation -ready to use-

as follows: Beta Valve DIN 3352 - DN 50

including connections. Flanged

socket adaptor with threaded connections.

Street top, carrier plate, stabilizer plate,

valve linkage unit.

1,00	pc.	_______________	__________________

8.2.	Drain Assemblies for Main pipes

Drain assemblies for main pipe lines

(DN 65-110) including coverage with a valve box

as well as all fittings, earth and additional

work.

1,00	pc.	_______________	__________________

Summe Titel 8. MAIN VALVES, VALVES,HYDRANT,DRILLINGS	__________________

Summe Lv  SPRINKLER IRRIGATION SYSTEM	__________________